Transmission protocol standard:

- Transmission speed = 9600.

- Data bits = 8.

- Stop bits = 1.

- Parity  - None.

- Data format = ASCII.

- Command definition:

"ENQ" = 05h

"ACK" = 06h

"NAK" = 15h

"SOH" = 01h

"STX" = 02h

"ETX" = 03h

"EOT" = 04h

"DC1" = 11h

"DC2" = 12h

"DC3" = 13h

"DC4" = 14h

Protocol:

| No | PC sends | Scale sends | Notes |
| --- | --- | --- | --- |
| 1 | ENQ |  | Inquiry about ready send data state |
| 2 |  | ACK | Scale answers "ACK" = YES or "NAK" = NO – in the situation PC repeats "ENQ" |
| 3 | DC1 |  |  |
| 4 |  | Data frame | Scale sends only weight data |

or:

| No | PC sends | Scale sends | Notes |
| --- | --- | --- | --- |
| 1 | ENQ |  | Inquiry about ready send data state |
| 2 |  | ACK | Scale answers "ACK" = YES or "NAK" = NO – in the situation PC repeats "ENQ" |
| 3 | DC2 |  |  |
| 4 |  | Data frame | Scale sends weight, unit price and total price data |

Data frame is as follows:

| No | Scale sends | Notes |
| --- | --- | --- |
| 1 | STX | Start of data frame |
| 2, n+1 | In turn: D1, ..., Dn | Next bytes (ASCII characters) data frame |
| n+2 | BCC | BCC = xor (logic sum) bytes from No. 2 to No. n+1 |
| n+3 | ETX | End of data frame |

Data frame for weight (RD1):

| Byte | Byte meaning | Notes |
| --- | --- | --- |
| D1 | STA | Sign "S" – for weight indication stabilized since 500 ms. Sign "U" for non yet weight indication stabilized |
| D2 | SIGN | Sign " " (Space) for weight equal or above “zero” Sign "-" for weight below “zero” |
| D3..D8 | In turn: C4, C3, KD, C2, C1, C0 | C4..C0 ASCII digits for weight (C0 – least significant digit) KD = "." – decimal dot sign (format: 99.999 kg) |
| D9, D10 | UN | Signs "kg" (D9 = "k" , D10 = "g") |

Data frame for total price (RD2) and unit price (RD3):

| Byte | Byte meaning | Notes |
| --- | --- | --- |
| D1..D8 | C6, C5, C4, C3, C2, KD, C1, C0 | C6..C0 ASCII digits for total price (or unit price) (C0 – least significant digit) |

Data frame for command "DC1"

| No. | Scale sends | Notes |
| --- | --- | --- |
| 1 | SOH | Start of transmission |
| 2 | Weight frame RD1 | Weight at the scale |
| 3 | EOT | End of transmission |

Data frame for command  "DC2"

| No. | Scale sends | Notes |
| --- | --- | --- |
| 1 | SOH | Start of transmission |
| 2 | Total price data frame RD2 | Total price calculated at the scale |
| 3 | Weight data frame RD1 | Weight at the scale |
| 4 | Unit price data frame RD3 | Unit price typed in from scale keypad |
| 5 | EOT | End of transmission |

In weight, unit price and total price digits leading zeros are replaced by Spaces (except unity digit (C2)). In the way digit 0 sis showed as "      0.00". Decimal dot position is constant for all numbers (like at the scale display). If for any number ensured overflow than all digits including sign "SIGN" have ASCII values "F".

Time intervals used by scale:

- Between bytes  100 s

- From PC inquiry to scale answer  10 ms (There is recommended 100..200 s, but no more then 1 ms).

Time intervals used by PC (ECR):

- There is enough use 1 stop bit.